BEFORE STARTING ENGINE

Preflight Inspection	COMPLETE

Passenger Briefing	COMPLETE

Seats, Belts, Shoulder Harness	ADJUST & LOCK

Fuel Selector Valve	BOTH

Electrical Equipment/Avionics 	OFF

Brakes	SET

Circuit Breakers	CHECK IN

# STARTING ENGINE

Avionics/Radio Master	OFF

Mixture	RICH

Carburetor Heat	COLD (IN)

Prime	AS REQUIRED (2-6)

Throttle	OPEN 1/8”

Master Switch	ON

Propeller Area	CLEAR

Ignition Switch	START

Oil Pressure	CHECK

Throttle	WARM UP @ 1000RPM

Mixture	Lean for Smooth Running

Avionics/Radio Master	ON

## BEFORE TAXI

Lights & Flashing Beacon	AS REQUIRED

Audio Panel	SET

DG	SET

Flaps	UP

Brakes	TEST

Weather Information	OBTAIN

## BEFORE TAKEOFF

Parking Brake	SET

Doors and Windows	CLOSE & LOCK

Flight Controls	FREE & CORRECT

Fuel Selector Valve	BOTH

Fuel Quantity	CHECK

Oil Temp	CHECK

Mixture	RICH

Throttle	1700 RPM

Magnetos	CHECK (125 max +- 50 difference)

Carburetor Heat	CHECK for RPM Drop

Engine Instruments & Ammeter	CHECK

Suction Gage	CHECK

Throttle	IDLE

Throttle Friction Lock	ADJUST

Elevator Trim	SET for T/O

Flight Instruments	CHECK & SET

Radios	SET

Transponder	ALTITUDE

Lights	AS DESIRED

Brakes	RELEASE

### TAKEOFF

#### NORMAL

Wing Flaps	UP

Carburetor Heat	COLD (IN)

Elevator Trim	SET

Mixture	MAX RPM (RICH)

Throttle	FULL OPEN

Elevator Control	LIFT NOSE WHEEL (55 KIAS)

Climb Speed	73 KIAS

SHORT FIELD TAKEOFF

Wing Flaps ………..Short field 0..……...Soft field 10

Carburetor Heat	COLD (IN)

Elevator Trim	SET

Brakes	APPLY

Mixture	MAX RPM (RICH)

Throttle	FULL OPEN

Brakes	RELEASE

Elevator Control	SLIGHTLY TAIL LOW

Climb Speed	59 KIAS

ENROUTE CLIMB

Airspeed	85 KIAS

Throttle	FULL OPEN

Mixture	RICH

CRUISE

Power	2200 – 2500 RPM

Elevator Trim	ADJUST

Mixture	LEAN

DESCENT

Fuel Selector Valve	BOTH

Power	AS DESIRED

Mixture	ADJUST

Carburetor Heat	FULL HEAT (OUT) AS REQD

BEFORE LANDING

Seats, Belts, Harnesses	SECURE

Fuel Selector Valve	BOTH

Mixture	RICH

Carburetor Heat	FULL HEAT (OUT)

LANDING

NORMAL LANDING

Airspeed	65 – 70 KIAS (flaps up)

Wing Flaps	AS DESIRED (below 85 KIAS)

Airspeed	60 – 65 KIAS (flaps down)

Touchdown	MAINS FIRST

Landing Roll	LOWER NOSEWHEEL GENTELY

Braking	MINIMUM REQUIRED

SHORT FIELD LANDING

Airspeed	65 – 70 KIAS (flaps up)

Wing Flaps	FULL DOWN

Airspeed	60 KIAS (until flare)

Throttle	REDUCE to Idle after clearing obstacle

Touchdown	MAINS FIRST

Landing Roll	LOWER NOSEWHEEL GENTELY

Braking	AS REQUIRED

Wing Flaps	RETRACT

GO-AROUND

Throttle	FULL OPEN

Carburetor Heat	COLD (IN)

Wing Flaps	20

Climb Speed	55 KIAS

Wing Flaps	10 (until obstacles cleared)

RETRACT after reaching safe alt. & 60 kt

AFTER LANDING

Carburetor Heat	COLD (IN)

Wing Flaps	UP

Mixture ……………………………………………Lean

Lights	As Required

SHUTDOWN

Brakes	SET

Avionics Master	OFF

Electrical Equipment & Lights	OFF

Mixture	IDLE CUTOFF

Ignition Switch	OFF

Master	OFF

Control Lock	INSTALL

Hobbs	RECORD

Aircraft	SECURE

Flight Plan	CLOSE

ENGINE FAILURE DURING TAKEOFF RUN

Throttle	IDLE

Brakes	APPLY

Wing Flaps	RETRACT

Mixture	IDLE CUT-OFF

Ignition Switch	OFF

Master Switch	OFF

ENGINE FAILURE IMMEDIATELY AFTER TAKEOFF

Airspeed	65 KIAS (flaps up) 60 KIAS (flaps down)

Mixture	IDLE CUT-OFF

Fuel Selector Valve	OFF

Ignition Switch	OFF

Wing Flaps	As Required

Master Switch	OFF

ENGINE FAILURE DURING FLIGHT

Airspeed	65 KIAS

Carb Heat	ON

Fuel Selector Valve	BOTH

Mixture	RICH

Ignition Switch	BOTH (or Start if prop stopped)

Primer	IN & LOCKED

Transponder	7700

Radio	121.5 MAYDAY!

EMERGENCY LANDING W/O POWER

Airspeed	65 KIAS (flaps up) 60 KIAS (flaps down)

Mixture	CUT-OFF

Fuel Selector Valve	OFF

Ignition Switch	OFF

Wing Flaps 	 As required (full recommended)

Master Switch	OFF

Doors 	UNLATCH PRIOR TO TOUCHDOWN

Touchdown	Slightly Tail Low (min. speed)

Brakes	APPLY HEAVILY

Max Weight  2300 lbs         Useful Load  802 lbs

Useful Load Full Fuel  544 lbs     Max Baggage  120 lbs

Oil Capacity 6 qts   Fuel Capacity 43 gal   Usable Fuel 40

Vso      41 KIAS	Vfe      85 KIAS	Downwind   75

Vs1      47 KIAS	Va       97 KIAS	Base           70

Vx        59 KIAS	Vno     128 KIAS	Final            65

Glide    65 KIAS	Vne     160 KIAS	Short           60

Vy        73 KIAS	X-wind 15 kts	TAS           110

Glenndale (8I3)

CTAF	122.90

Grissom (KGUS)

APPROACH	121.05(East) 123.85(West)

TOWER	133.7

GROUND	128.425

ATIS	108.45

CLEARANCE	120.0

Indy Exec (KTYQ)

CTAF	123.05

AWOS	120.725

(317-796-3154)

Kokomo (KOKK)

CTAF	123.0

AWOS	128.675

(765-452-3014)

Logansport (KGGP)

CTAF	122.8

AWOS	118.5

(574-737-7310)

Marion (KMZZ)

CTAF	122.7

AWOS	108.6

(765-674-1317)

Muncie (KMIE)

TOWER (CTAF)	120.1

GROUND	121.9

ATIS	133.25

UNICOM	122.95

Noblesville (I80)

CTAF	122.9

Sheridan (5I4)

CTAF	123.075

Westfield (I72)

CTAF	122.90