# CESSNA N9924B

DO NOT RELY ON THIS INFORMATION FOR AIRCRAFT OPERATIONS.  ALWAYS USE THE OFFICIAL PILOT OPERATING HANDBOOK OR AIRCRAFT MANUAL.

# BEFORE STARTING ENGINE

Preflight Inspection	COMPLETE

Passenger Briefing	COMPLETE

Seats, Belts, Shoulder Harness	ADJUST & LOCK

Fuel Selector Valve	BOTH

Brakes	SET

Circuit Breakers	CHECK IN

# STARTING ENGINE

Avionics/Radio Master	OFF

Cowl Flaps	OPEN

Mixture	RICH

Prop	HIGH RPM (IN)

Carburetor Heat	COLD (IN)

Prime	AS REQUIRED (2-4)

Throttle	OPEN 1/4”

Master Switch	ON

Propeller Area	CLEAR

Ignition Switch	START

Oil Pressure	CHECK

Throttle	WARM UP @ 1000RPM

Mixture	Lean for Smooth Running

Avionics/Radio Master	ON

## BEFORE TAXI

Lights & Flashing Beacon	AS REQUIRED

Audio Panel	SET

DG	VERIFY

Flaps	UP

Brakes	TEST

Weather Info	OBTAIN

## BEFORE TAKEOFF

Parking Brake	SET

Doors and Windows	CLOSE & LOCK

Flight Controls	FREE & CORRECT

Fuel Selector Valve	BOTH

Fuel Quantity	CHECK

Oil Temp	CHECK

Mixture	RICH

Prop	High RPM

Throttle	1800 RPM

Magnetos	CHECK (150 max +- 50 difference)

Carburetor Heat	CHECK for RPM Drop

Engine Instruments & Ammeter	CHECK

REDUCE THROTTLE TO 1500 RPM

CYCLE PROP *ONCE* THEN RETURN TO HIGH RPM

Throttle	IDLE

Throttle Friction Lock	ADJUST

Elevator and Rudder Trim	SET for T/O

Annunciator Lights 	CHECK

Aux Fuel Pump	CHECK

Gear Warning System	CHECK

Flight Instruments	CHECK & SET

Radios/GPS	SET

Transponder	ALTITUDE

Wing Flaps	SET for Takeoff

Lights	AS DESIRED

Brakes	RELEASE

### NORMAL TAKEOFF

Auto Pilot	OFF

Wing Flaps	UP

Carburetor Heat	COLD (IN)

Elevator Trim 	SET

Cowl Flaps	OPEN

Mixture 	RICH

Prop	HIGH RPM

Throttle	FULL OPEN

Elevator Control	LIFT NOSE WHEEL (55 KIAS)

Brakes	Momentarily Apply ONCE AIRBORNE

Landing Gear	RETRACT

Climb Speed	84 KIAS

SHORT FIELD TAKEOFF

Auto Pilot	OFF

Wing Flaps	(Soft field 10°)(Short field 0°)

Carburetor Heat	COLD (IN)

Elevator Trim 	SET

Cowl Flaps	OPEN

Brakes	APPLY

Mixture	RICH

Throttle	FULL OPEN

Brakes	RELEASE

Elevator Control	SLIGHTLY TAIL LOW

Climb Speed	63 KIAS

Brakes	Momentarily Apply ONCE AIRBORNE

Landing Gear	After Clearing Obstacles RETRACT

ENROUTE CLIMB

Cowl Flaps	OPEN

Airspeed	85 KIAS

Throttle	Maintain 25” MP

Prop	2500 RPM

Mixture	RICH

CRUISE

Throttle	15”-24” MP

Prop	2100 – 2400 RPM

Elevator Trim	ADJUST

Mixture	LEAN

Cowl Flaps	Closed-Open as Needed

DESCENT

Fuel Selector Valve	BOTH

Throttle	15”-24” MP

Prop	2100-2500 RPM

Mixture	ADJUST

Carburetor Heat	FULL HEAT (OUT) AS REQD

Cowl Flaps	As Needed

BEFORE LANDING

Auto Pilot	OFF

Seats, Belts, Harnesses	SECURE

Fuel Selector Valve	BOTH

Landing Gear	DOWN/GREEN LIGHT/CHECK

Mixture	RICH

Prop	High RPM

Carburetor Heat	FULL HEAT (OUT)

NORMAL LANDING

Flaps	AS DESIRED (below 85 KIAS)

Airspeed	65-70 KIAS (flaps down)

Touchdown	MAINS FIRST

Landing Roll	Lower Nose Wheel Gently

Braking	MINIMUM REQUIRED

SHORT FIELD LANDING

Flaps	FULL DOWN

Airspeed	63 KIAS (until flare)

Throttle	REDUCE to Idle After Clearing Obstacle

Touchdown	MAINS FIRST

Landing Roll	Lower Nose Wheel Gently

Braking	AS REQUIRED

Wing Flaps	RETRACT

GO-AROUND

Throttle	FULL OPEN

Prop	HIGH RPM (IN)

Carburetor Heat	COLD (IN)

Wing Flaps	20 (Immediately)

Climb Speed	65 KIAS

Wing Flaps	10° (until obstacles cleared)

Retract gear/flaps at safe altitude & climb	84 KIAS

AFTER LANDING

Lights	AS REQUIRED

Carburetor Heat	COLD (IN)

Prop	HIGH RPM (IN)

Mixture	LEAN

Wing Flaps	UP

Trim	Set for Takeoff

Cowl flaps	OPEN

SHUTDOWN

Brakes	SET

Avionics Master	OFF

Electrical Equipment & Lights	OFF

Mixture	IDLE CUTOFF

Ignition Switch	OFF

Master	OFF

Control Lock	INSTALL

Hobbs	RECORD

Aircraft	SECURE

Flight Plan	CLOSE

ENGINE FAILURE DURING TAKEOFF RUN

Throttle	IDLE

Brakes	APPLY

Wing Flaps	RETRACT

Mixture	IDLE CUT-OFF

Ignition Switch	OFF

Master Switch	OFF

ENGINE FAILURE IMMEDIATELY AFTER TAKEOFF

Airspeed	70 KIAS (flaps up) 65 KIAS (flaps down)

Mixture	IDLE CUT-OFF

Fuel Selector Valve	OFF

Ignition Switch	OFF

Wing Flaps 	As Required

Master Switch	OFF

Max Weight  2658 lbs         Useful Load  949.11 lbs

Useful Load Full Fuel  553.11 lbs

Oil Capacity 8 qts   Fuel Capacity 66 gal   Usable Fuel 62

Vso      42	Vfe        130	Downwind     75

Vs1      47 	Va       89-106	Base              70

Vx        67 (68*)	Vno       145	Final              65

Glide    67	Vne       164	Short             63

Vy        84 (77*)	Vlo        140	TAS             130

*Note: @10,000ft	Vle        164	X-wind           15

ENGINE FAILURE DURING FLIGHT

Airspeed	75 KIAS

Carb Heat	ON

Fuel Selector Valve	BOTH

Mixture	RICH

Aux Fuel Pump	ON

Ignition Switch	BOTH (or Start if Prop Stopped)

Primer	IN & LOCKED

Transponder	7700

Radio	121.5 MAYDAY!

EMERGENCY LANDING W/O POWER

Airspeed	75 KIAS (flaps up) 65 KIAS (flaps down)

Mixture	CUT-OFF

Fuel Selector Valve	OFF

Ignition Switch	OFF

GEAR	DOWN (UP for rough or soft terrain)

Wing Flaps 	As Required (30° Recommended)

Master Switch	OFF

Doors 	UNLATCH PRIOR TO TOUCHDOWN

Touchdown	Slightly Tail Low (min. speed)

Brakes	APPLY

Glenndale (8I3)

CTAF	122.90

Grissom (KGUS)

APPROACH	121.05(East) 123.85(West)

TOWER	133.7

GROUND	128.425

ATIS	108.45

CLEARANCE	120.0

Indy Exec (KTYQ)

CTAF	123.05

AWOS	120.725

(317-796-3154)

Kokomo (KOKK)

CTAF	123.0

AWOS	128.675

(765-452-3014)

Logansport (KGGP)

CTAF	122.8

AWOS	118.5

(574-737-7310)

Marion (KMZZ)

CTAF	122.7

AWOS	108.6

(765-674-1317)

Muncie (KMIE)

TOWER (CTAF)	120.1

GROUND	121.9

ATIS	133.25

UNICOM	122.95

Noblesville (I80)

CTAF	122.9

Sheridan (5I4)

CTAF	123.075

Westfield (I72)

CTAF	122.90