PREFLIGHT

N2018E – Aeronca Champ 7AC

Use Pilot's Operating Handbook for official and complete list

INTERIOR

- Electrical Master - OFF

- Fuel – ON (to allow better sumping)

- Carb Heat – OFF

- Mags – OFF (full counterclockwise)

- Throttle – CLOSED

- Flight Log – Record engine tach hours

- AROW – Airworthiness, Registration, Operating Limits, Weight / Balance

- Baggage Weight – Maximum 20 lbs

- Radios and Intercom – CHECK (then as desired)

- Lights and other electrical – OFF

- Fuel Quantity – CHECK

- Altimeter – SET (AWOS or field elevation)

- Compass – CHECK

- Other Items – Headsets, Charts, Batteries, Tie Downs

EXTERIOR

1. FUEL

- Fuel Tank(s) – CHECK QUANTITY (touch or dip stick)

- Filler Cap(s) – SECURE

2. ENGINE

- Engine Oil – CHECK (3 quarts minimum)

- Fuel Sump – DRAIN (inspect for water or debris)

- Engine Compartment – INSPECT for leaks, chafing, etc.

- Cowl – SECURE both left and right sides

3. WALK AROUND

- Oil or Fuel Puddles – CHECK

- Attachment Points – CHECK

- Hinges – CHECK

- Covering and Surfaces – CHECK including underneath

- Freedom of Ailerons, Elevator, and Rudder – CHECK

- Tail Wires – CHECK proper tension

- Tailwheel – CHECK including no excess wobble

- Trim – CHECK including control tension

- Tires – CHECK inflation 15-20 lbs, strut, brake condition

- Propeller – CHECK for general condition and nicks

- Induction Air Intake – CLEAR

- Air Filter – In good condition

- Exhaust – Both present and secure

- Pitot / Static – CHECK

- Windshield – CLEAN

AIRPORTS

Westfield (I72)	122.900		932'

Rwy 18/36 is 3000 x 100 turf 120.725          TYQ

Executive (TYQ)	123.050		922'

Rwy 18 / 36 is 5500 x 100	         120.725 AWOS

Sheridan (5I4)	123.075		936'

Rwy 05 / 23 is 3760 x 50	            120.725 TYQ

Rwy 09 / 27 is 3387 x 88 turf

Noblesville (I80)	122.900		821'

Rwy 09 RH/27 is 3580 x 100 turf    119.375 UMP

Glenndale (8I3)	122.900		850'

Rwy 18/36 is 2048 x 110 turf          113.500 OKK

Metro (UMP)		123.000		811'

Rwy 15 / 33 is 4004 x 100	          119.375 AWOS

Kokomo (OKK)	123.000		830'

Rwy 05 / 23 is 6001 x 150	          113.500 AWOS

Rwy 14 / 32 is 4002 x 150

Frankfort (FKR)	123.000		860'

Rwy 09 / 27 is 5000 x 75	          124.325 AWOS

Rwy 04 / 22 is 32527 x 70

AIRSPEEDS

Vs stall				  38 MPH

Vx best angle			  50 MPH

Vy best rate			  55 MPH

Best Glide			  60 MPH

Va maneuvering speed		  85 MPH

Vne never exceed			135 MPH

Maximum Crosswind		  12 MPH

SPECIFICATIONS

Engine Continental C-85	  85 HP at 2575 RPM

Fuel Consumption Cruise		      5.4 GPH

Main Tank			13.0 Gallons

Oil Capacity			      4 Quarts

Gross Weight		1320.0 lbs (check)

Empty Weight		  889.5 lbs (check)

Useful Load		  430.5 lbs (check)

Rate of Climb (Max)	   620 fpm (estimated)

Take Off (2000 ft alt, 95 deg F, no wind, turf)

Ground Roll		      565 ft (estimated)

Clear 50 ft                              1350 ft (estimated)

Landing (2000 ft altitude, no wind, turf)

Ground Roll	        275 ft (with good brakes)

Clear 50 ft	                  1150 ft (estimated)

NOTE – Land before main fuel tank bobber has less than two inches showing.

STARTING

N2018E – Aeronca Champ 7AC

BEFORE STARTING

- Preflight Inspection – COMPLETE

- Winds and Weather – CHECKED

- Seats Belts – SECURE

- Passengers – BRIEF:
(a) seat belts, (b) doors and windows, (c) headset and intercom, (d) point out all other aircraft

- Loose items – STORED including unused seat belts

- Tail – TIED DOWN as required

- Prop Area – CLEAR including prop blast area behind

STARTING ENGINE

COLD START (preheat below 45 deg)

- Mags – OFF

- Throttle – CLOSED

- Fuel – ON

- Carb Heat – COLD

- Prime – Pull prop through five blades

- Throttle – IDLE (trying to achieve 600 RPM)

- Mags – ON BOTH (three “clicks”)

- Brakes – SET

- Battery Switch – ON

- Starter Button – PUSH (not more than 6 seconds)

HOT START (engine warm to the touch)

- Fuel – ON

- Carb Heat – COLD

- Throttle – IDLE (trying to achieve 600 RPM)

- Mags – ON BOTH (three “clicks”)

- Brakes – SET

- Battery Switch – ON

- Starter Button – PUSH (not more than 6 seconds)

FLOODED ENGINE

- Fuel – ON

- Carb Heat – COLD

- Throttle – FULL OPEN

- Mags – ON BOTH (three “clicks”)

- Brakes – SET (really good)

- Battery Switch – ON

- Starter Button – PUSH (not more than 6 seconds)

- Throttle – REDUCE TO IDLE as engine clears

AFTER STARTING

- Oil Pressure – CHECK

- Warm-Up – 1000 RPM

- Time – Note Start Time

- Radios and Intercom – ON

- Headsets – ON

- Other electrical devices – ON

BEFORE TAXI

- Seat Belts – SECURE

- Door – CLOSED

- Radio – Listen and make appropriate calls

TAXI

- Brakes – RELEASE and CHECK

- Controls During Taxi:
a) Headwind – Climb and bank into headwind
b) Tailwind – Dive and bank away from tailwind

EMERGENCY PROCEDURES

ENGINE FAILURE

- Airspeed – PITCH for best glide 60 MPH (wings level)

- Trim – SET slide all the way back which is full nose up

- Landing Site – PICK and go directly to approach end

- Radio – Call MAYDAY (use 121.5 if able)

RESTART, ROUGH, or LOSS OF POWER

- Fuel – Confirm ON and quantity

- Aux Fuel – ON (if available)

- Prime – IN and LOCKED

- Mags – Try LEFT and RIGHT then BOTH

- Carb Heat – CHANGE IT ON or OFF

- Throttle – CHANGE POSITION

LANDING

- Spiral Down – AS APPROPRIATE

- Slip – AS APPROPRIATE

- Airspeed – Best glide 60 MPH (wings level)

- Seat Belts – SECURE make them extra tight

- Fuel – OFF

- Aux Fuel – OFF (if available)

- Throttle – CLOSED

- Mags – OFF

- Electrical Master – OFF

- Doors – UNLATCHED

- Touchdown – Normal 3-point landing with full stall

- Exit – IMMEDIATELY

DEPARTING

N2018E – Aeronca Champ 7AC

RUN UP

- Park – INTO THE WIND (if practical to do so)

- Brakes – HOLD

- Throttle – 1700 RPM

- Magnetos – CHECK (100 max drop, 50 max difference)

- Carb Heat – CHECK (noticeable drop 25 to 150 RPM)

- Oil Pressure – CHECK (green arc 10 to 48 PSI)

- Oil Temperature – CHECK (green arc 90 to 220 °F)

- Throttle – IDLE then as desired

BEFORE TAKEOFF

- Controls – FREE and CORRECT

- Doors and Windows – SECURE

- Seat Belts – SECURE

- Altimeter and Heading – CHECK

- Radio, Intercom, Navigation Devices – CHECK

- Carb Heat – OFF

- Trim – NEUTRAL

- Radio – Make appropriate calls including direction of flight

- Time – Note departure time for navigation purposes

TAKEOFF

- Carb Heat – OFF

- Trim – NEUTRAL

- Stick Position – AS APPROPRIATE

- Throttle – FULL OPEN

- Engine RPM – Above 2300 RPM

- Oil Pressure and Temperature – CHECK

- Climb – Vx 50 MPH or Vy 55 MPH (sea level)

CRUISE

- Throttle – 2200 to 2450 RPM

- Airspeed – 80 to 95 MPH typical in level flight

- Oil Pressure and Temperature – MONITOR

- Fuel – MONITOR

- Carb Heat – ON if engine is below 2000 RP

BEFORE LANDING

- Seat Belts – SECURE

- Carb Heat – ON

- Trim – AS DESIRED

- Power – AS REQUIRED

AFTER LANDING

- Carb Heat – OFF

- Trim – NEUTRAL

- Fuel – Refuel as necessary

SHUTDOWN

- Carb Heat – OFF

- Throttle – IDLE

- Radios and Intercom – OFF

- Lights and other electrical – OFF

- Electrical Master – OFF

- Fuel – OFF

- Mags – OFF

- Time – Note stop time

- Flight Log – Record engine tach hours and flight time

- Aircraft – SECURED from wind and weather

- Personal Items – CHECK got everything