BEFORE STARTING ENGINE

Preflight Inspection	COMPLETE

Passenger Briefing	COMPLETE

Seats, Belts, Shoulder Harness	ADJUST & LOCK

Fuel Selector Valve	PROPER TANK (lowest)

Autopilot 	OFF

Electrical Equipment/Avionics 	OFF

Brakes	SET

Circuit Breakers	CHECK IN

# STARTING ENGINE

Avionics/Radio Master	OFF

Mixture	RICH

Carburetor Heat	COLD (IN)

Prime	AS REQUIRED (2-6)

Throttle	OPEN 1/8”

Master Switch	ON

Fuel Pump	ON and CHECK pressure

Propeller Area	CLEAR

Ignition Switch	START

Oil Pressure	CHECK

Throttle	WARM UP @ 1000RPM

Fuel Pump	OFF

Mixture	Lean for Smooth Running

Avionics/Radio Master	ON

## BEFORE TAXI

Lights & Flashing Beacon	AS REQUIRED

Audio Panel	SET

DG	SET

Flaps	UP

Weather Information	OBTAIN

Brakes	TEST

## BEFORE TAKEOFF

Parking Brake	SET

Doors and Windows	CLOSE & LOCK

Flight Controls	FREE & CORRECT

Fuel Selector Valve	PROPER TANK (fullest)

Fuel Quantity	CHECK

Oil Temp	CHECK

Mixture	RICH

Throttle	2000 RPM

Magnetos	CHECK (125 max +- 50 difference)

Carburetor Heat	CHECK for RPM Drop

Engine Instruments & Ammeter	CHECK

Suction Gage	CHECK

Throttle	IDLE then 1000 RPM

Throttle Friction Lock	ADJUST

Elevator and Nose Trim	SET for T/O

Fuel Pump	ON

Flight Instruments	CHECK & SET

Radios	SET

Transponder	ALTITUDE

Lights	AS DESIRED

Brakes	RELEASE

### TAKEOFF

#### NORMAL

Wing Flaps	UP

Carburetor Heat	COLD (IN)

Elevator Trim	SET

Mixture	MAX RPM (RICH)

Throttle	FULL OPEN

Elevator Control	LIFT NOSE WHEEL (55 MPH)

Climb Speed	85 MPH

SHORT FIELD TAKEOFF

Wing Flaps ………..Short field 0..……...Soft field 25

Carburetor Heat	COLD (IN)

Elevator Trim	SET

Brakes	APPLY

Mixture	MAX RPM (RICH)

Throttle	FULL OPEN

Brakes	RELEASE

Elevator Control	SLIGHTLY TAIL LOW

Climb Speed	74 MPH

ENROUTE CLIMB

Airspeed	100 MPH

Throttle	FULL OPEN

Mixture	RICH

Fuel Pump	ON

CRUISE

Power	2200 – 2500 RPM

Elevator Trim	ADJUST

Mixture	LEAN

Fuel Pump	OFF

DESCENT

Fuel Selector Valve	PROPER TANK (fullest)

Power	AS DESIRED

Mixture	ADJUST

BEFORE LANDING

Seats, Belts, Harnesses	SECURE

Autopilot 	OFF

Fuel Selector Valve	PROPER TANK (fullest)

Fuel Pump	ON

Mixture	RICH

Carburetor Heat	AS REQUIRED

LANDING

NORMAL LANDING

Wing Flaps	AS DESIRED (below 115 MPH)

Airspeed	80 – 85 MPH (flaps down)

Touchdown	MAINS FIRST

Landing Roll	LOWER NOSEWHEEL GENTELY

Braking	MINIMUM REQUIRED

SHORT FIELD LANDING

Wing Flaps	FULL DOWN

Airspeed	80 MPH (until flare)

Throttle	REDUCE to Idle after clearing obstacle

Touchdown	MAINS FIRST

Landing Roll	LOWER NOSEWHEEL GENTELY

Braking	AS REQUIRED

GO-AROUND

Throttle	FULL OPEN

Carburetor Heat	COLD (IN)

Wing Flaps	RETRACT SLOWLY

Climb Speed	74-85 MPH

AFTER LANDING

Carburetor Heat	COLD (IN)

Wing Flaps	UP

Mixture ……………………………………………Lean

Fuel Pump	OFF

Lights	As Required

SHUTDOWN

Brakes	SET

Avionics Master	OFF

Electrical Equipment & Lights	OFF

Mixture	IDLE CUTOFF

Master	OFF

Ignition Switch	OFF

Control Lock	INSTALL

Hobbs	RECORD

Aircraft	SECURE

Flight Plan	CLOSE

ENGINE FAILURE DURING TAKEOFF RUN

Throttle	IDLE

Brakes	APPLY

WITH FIRE

Mixture	IDLE CUT-OFF

Ignition Switch	OFF

Master Switch	OFF

ENGINE FAILURE IMMEDIATELY AFTER TAKEOFF

Airspeed	80 MPH (flaps up) 70 MPH (flaps down)

Mixture	IDLE CUT-OFF

Fuel Selector Valve	OFF

Ignition Switch	OFF

Wing Flaps	As Required

Master Switch	OFF

ENGINE FAILURE DURING FLIGHT

Airspeed	80 MPH

Carb Heat	ON

Fuel Selector Valve	SWITCH TANKS

Fuel Pump	ON

Mixture	RICH

Ignition Switch	BOTH (or Start if prop stopped)

Primer	IN & LOCKED

Transponder	7700

Radio	121.5 MAYDAY!

EMERGENCY LANDING W/O POWER

Airspeed	80 MPH (flaps up) 70 MPH (flaps down)

Mixture	CUT-OFF

Fuel Selector Valve	OFF

Ignition Switch	OFF

Wing Flaps 	 As required (full recommended)

Master Switch	OFF

Door 	UNLATCH PRIOR TO TOUCHDOWN

Touchdown	Slightly Tail Low (min. speed)

Brakes	APPLY HEAVILY

Max Weight  2400 lbs         Useful Load  972.5 lbs

Useful Load Full Fuel  678.5 lbs     Max Baggage  200 lbs

Oil Capacity 8 qts   Fuel Capacity 50 gal   Usable Fuel 49

Vso      57 MPH	Vfe      115 MPH	Downwind   90

Vs1      67 MPH	Va       129 MPH	Base           85

Vx        74 MPH	Vno     140 MPH	Final            80

Glide    80 MPH	Vne     171 MPH	Short           76

Vy        85 MPH	X-wind 15 kts	TAS           143

Glenndale (8I3)

CTAF	122.90

Grissom (KGUS)

APPROACH	121.05(East) 123.85(West)

TOWER	133.7

GROUND	128.425

ATIS	108.45

CLEARANCE	120.0

Indy Exec (KTYQ)

CTAF	123.05

AWOS	120.725

(317-796-3154)

Kokomo (KOKK)

CTAF	123.0

AWOS	128.675

(765-452-3014)

Logansport (KGGP)

CTAF	122.8

AWOS	118.5

(574-737-7310)

Marion (KMZZ)

CTAF	122.7

AWOS	108.6

(765-674-1317)

Muncie (KMIE)

TOWER (CTAF)	120.1

GROUND	121.9

ATIS	133.25

UNICOM	122.95

Noblesville (I80)

CTAF	122.9

Sheridan (5I4)

CTAF	123.075

Westfield (I72)

CTAF	122.90